An Analysis of Stress in Guy Wire Systems

William Rynone, Ph.D., P.E.

(WIND_LD2.DOC)

Oveview...

Due to our litigious society, I would like to state that my area of expertise is sparks and arcs rather than snaps and cracks.  Therefore the following should be used as an educational tool rather than as an analysis or design format for specific tower installations.

My interest in antenna tower mechanical considerations was precipitated via my HAM Club's interest in erecting one or two additional towers.  It appears that tower and antenna manufacturers generally provide adequate information regarding tower installation parts and procedures, such as tower mounting base materials and construction, guy wire size and pre-loading, anchor methods, etc.  The recommended guy wire specifications are given without mentioning the actual stress that the wires will experience under various load conditions.  For example, if a tower manufacturer specifies 3,000# guy wire cable, I wondered what the actual stress would be.   Typical cable specifications are those listed below in a table from literature that is supplied by the Wire Rope Corporation Of America.

Page 22-5 of the ARRL Antenna Book, 15th EDITION contains a brief description of guy wire materials, including, "At rated working load tension, dry manila rope stretches about 5%, while nylon rope stretches about 20%.  Antenna wire is suitable for guys, particularly the copper clad steel types.  Solid galvanized steel wire is also  widely used for guying.  This wire has approximately twice the tension ratings of similar sizes of copper-clad wire, but is more susceptible to corrosion.  Stranded galvanized wire sold for guying TV masts is also available for light-duty applications, but is susceptible to corrosion."  Since a wire rope manufacturer usually has a minimum order length or dollar value, and since the average HAM will only require small lengths of guy wire, buying directly from a manufacturer is usually not possible.  However, some tower suppliers, also sell ancillary equipment or parts to aid in the tower installation.  e.g. From the Texas Tower's web site, both galvanized steel and a non-conductive cable, Phillystran, in various tension ratings and lengths is available.  Also the minimum length of galvanized steel cable that they will sell is 100 feet.  From the Wirerope Works Structural Strand And Wire Rope Catalog, a HAM would also consider the following to be pertinent when installing a guyed tower.  "Assuming the applied tension does not exceed the elastic limit, structural strand and wire rope return to their original length once tension is released.  Constructural stretch, on the other hand, results in a permanent set or increase in length."

General Analysis of Guy Wire Stress...

There are various forces creating tension in guy wires.  e.g. Wind load on the tower and mounted antennas, pre-load in the guy wire, and to a lesser extent, wind load on the guy wire itself and the weight of the guy wire.

The tower experiences various loads.  These loads manifest themselves in compression, buckling and shear.  Compression is due to the downward vertical force from the guy wires (due to all of the above mentioned forces on the guy wire), the weight of the tower and the weight of the antennas.  Shear is developed due to side loads on the tower and mounted antennas.  It is possible that due to varying wind loads and variations in guy wire tensioning, that at a particular instant, one section of an antenna tower will be tending to bend in one direction while another section will be tending to bend in the opposite direction.

I estimate that investigating all of these loads would generate sufficient data and descriptors to create a textbook (which may already exist).

The following is just the analysis of the load on the guy wires due to wind load on the tower and antenna.  Usually a single set of guy wires is employed for towers of height equal to or less than 40 feet.  To minimize the complexity of this analysis, this condition will be analyzed with only a single layer dipole antenna.  Also, tower twisting due to an asymmetric antenna load is not examined.

The associated computer program enables the user to input various tower and antenna dimensions and a wind speed.  The program then computes the guy wire tension for various guy wire radial distances from the tower base.  The program neglects the contribution to the total guy wire tension due to wind loading on the guy wire and weight of the guy wire.

It is appropriate to mention that the guy wire pre-load is a trade-off.  The greater the pre-load, the less tower sway due to varying wind speeds and directions, but at the sacrifice of a greater compression load on the tower and tension in the guys.  The ARRL Antenna Book contains the following, "Most troubles encountered in mast guying are a result of pulling the guy wires too tight(ly).  Guy wire tension should never be more than necessary to correct for obvious bowing or movement under wind pressure."

To perform this analysis, the following tasks must be accomplished... Evaluate the,

1.	pressure developed on the antenna due to the wind

2.	transverse antenna tower torque due to the antenna force

3.	transverse antenna tower torque due to wind pressure on the horizontal members of the supporting web

4.	transverse torque due to wind pressure on the three vertical members

5.	transverse torque due to wind pressure on the diagonal members of the supporting web

6.	total torque and the resulting horizontal force at the top of the tower where the guy wires are attached

7.	force generated in the guy wire for various declination angles

Equation Symbols:

Antenna -

HA = Height of Antenna (ft)

ED = Element Diameter (ft)

EL = Antenna element Length (ft)

FA = Force that the Antenna applies to the antenna tower (#)

TA = Torque due to the antenna (ft*#)

Vertical members -

VD = Vertical member Diameter (ft)

TVM= transverse Torque due to a Vertical Member (ft*#)

HT = Height of Tower (also equals vertical member height) (ft)

Web members -

WW = Web Width (horizontal space between vertical members) (ft)

WH = Web Height (vertical space between adjacent webs) (ft)

RD = Rebar Diameter (ft)

THW= Torque due to all Horizontal Web members (ft*#)

TDM=Torque due to Diagonal Members (ft*#)

HWM = Horizontal Web Member

Tower -

GWFP = Guy Wire Force Perpendicular to tower (#)

GWTF = Guy Wire Tension Force (#)

V = Wind Speed (m.p.h.)

P = Wind Pressure applied to antenna and tower (#/ft2)

R = Radius of guy wire anchors to tower base (ft)

TT = Total transverse Torque due to wind (ft*lb)

Cd = Coefficient of drag

Analyses:

1. Pressure developed on the antenna due to wind (This analysis only deals with a steady wind)...

The Wirerope Works literature has an old publication that gives the pressure developed on square and round smoke stacks:  P = 0.0042 V2  (Square) and P = 0.0025 V2 (Round).  The equation for a round smoke stack is similar to one given in EIA-222-F (2)  for cylindrical surfaces.

The force developed on one antenna element due to a wind that is perpendicular to the element, is given by FA = A * P * Cd.  Where Cd = 1.2 for long cylinders (i.e. antenna beam elements).  Re-writing:  FA = EL * ED * P * Cd

2.  Transverse antenna tower torque due to antenna force....

TA=M*FA*HA

µ §    (1)

Where M= the number of horizontal antenna elements

3.  Transverse antenna tower torque due to wind pressure on the horizontal members of the supporting web...

The number of webs, NW is determined by dividing the tower height by the web height.   NW = HT/WH

Each web cell has one horizontal element.  See below.

The wind pressure on each horizontal web member is given by P = 0.0025 V2

The force on each web member due to the pressure is given by:

FW = Area*Pressure*Coefficient of Drag = WW*RD*P*Cd

The torque is the force times the lever arm:  Where each Horizontal Web Member force is equal but the lever arm (as measured from the antenna base), increments by the web height.

µ §

µ §

Referring to the diagram below, you will note that the sum of the forces generated on the two rear members is equal to the force on the member facing the wind.

Therefor for three sets of webs,

µ §         (2)

4.	Transverse torque due to wind pressure on the three vertical members...

Analyzing the torque due to one vertical member, the following drawing is a depiction of the pressure on a single vertical member.

The incremental torque due to the wind pressure is given by:

µ §

Where:

y = Height from antenna base to an incremental area of the vertical member

dF = An incremental force due to wind pressure on an incremental area of the vertical member

dA = Incremental Area

Solving the above:

µ §

µ §

Where P = K * V2  (From Bethlehem Steel Wire Rope Applications Notes)

Where K = 0.0042 for flat stacks and 0.0025 for circular stacks.

For all three members,

µ §           (3)

5.	Transverse torque due to wind pressure on the diagonal members of the supporting web...

The total torsional effect of discrete diagonal members of each web cell may be treated as a single long ribbon that extends from the ground to a height equal to the tower height.  From the drawing for a cross-section of a diagonal rebar member,

we note that,

µ §

Referring to a diagonal section of the webbing,

µ §

However,

µ §

From the above,

µ §

For three sets of webs,

µ §       (4)

At this point, the total tangential torque is simply the sum of each of the individual torques: TT = TA + THWM + TVM + TDIAG

The perpendicular force applied to the top of the tower by a guy wire attached at the top may be calculated by recalling that the torque is equal to the product of force times lever arm:

µ §

Solving for the force:

µ §

Also the guy wire length may be calculated:

µ §

Also, to determine the guy wire tension force, GWTF:

µ §

This formula is incorporated into a computer program, WIND_LOD.EXE  that enables the user to input wind speed, tower height, tower width and effective cross sectional area.  The resulting output data consists of guy wire tension versus various distances between the tower base and guy wire anchor point.  The following is a sample of the output data.

The following graph is included to emphasise the importance of utilizing an adequate spacing between the antenna base and the guy wire anchor location.  The reader will note that the guy wire stress increases exponentially as the spacing decreases.  Some articles pertaining to guy wire installation, recommend that the minimum spacing be equal to or greater than 1/2 the antenna height (Assuming that the guy wire is attached to the top).  Referring to the graph where the antenna height is 40 feet, you will note that for spacing of less than 20 feet, the stress begins to increase rapidly.

The computer program is available from the author for free.  Send a blank diskette or CD to W. Rynone, Ph.D., P.E., Rynone Engineering, Inc., P.O. Box 4445, Annapolis, Maryland 21403.  I will return the diskette or CD with a copy of WIND_LOD.EXE.  Be certain to include your name and return address.

-------------------------------------------------------------------------------

References:

(1) "Guying of Stacks", Wire Rope Works - Bethlehem Steel

(2) "Array Solutions", Kurt Andress, K7NV

(3) "THE ARRL ANTENNA BOOK, 15th EDITION"

Acknowledgments:

Mr. Norm Chipps made practical suggestions, Mr. Jonathan Girard, Engineer, Rohn Products supplied technical information.  Mr. Oscar Ramsey reviewed the text.  Mr. Tom Hughes and Mr. Tom Secules of Wirerope Works kindly supplied technical information.  Tim Klein of Wire Rope Corporation of America discussed technical aspects of wire rope.