;

;

; ************************* LPDDS VER 3.00**********************************

;

; * Low Power Signal Generator (VFO) with Direct Digital Synthesis

;  FOR A SINGLE BAND with 12 MHz IF and offset (40 METERS)

; *  Edited by David B. Harrison W6IBC

; *  Version 3.00 for PIC16F819

; *  November 8, 2004

; ***************************************************************************

; Description:

; This is the control program for a LOW POWER DDS VFO built with an

; AD9834 low power DDS chip, an electromechanical shaft encoder,

; a push button switch and some form of display, currently 16 char by 2 line

; LCD module with Hitachi controller chip.  This program uses polling to check

; the shaft encoder.  Interrupts are not presently used, but would be used if

; a keyer program for CW transmit were added.

;

; Features:

;

; +5 volt 16 char by 2 line LCD display module with Hitachi chip.

;

; Encoder push button (PB-1) selects incremental step size: choices are

; 1kHz, 10kHz, 10Hz and 100Hz in a step cycle, based on a counter and a

; computed goto routine. Initially 1kHz.

;

; CALIBRATE  is entered if the calibrate pb_2 push button is pressed during

; power on. The display is set to 10,000.000 CAL and remains fixed, even as

; adjustments are being made. If the push button is held pressed, then turning

; the shaft encoder will increase or decrease the value "osc" used to

; calculate the DDS control word.  An external frequency counter on the DDS

; output is required to observe this adjustment. To exit calibrate mode,

; release the calibrate push button and turn the shaft encoder one more time.

; The calibrated value of "osc" will then be stored in EEPROM memory.

;

; RESET.  Hard reset is available by depressing PB-3 on DDS board, hard wired

; to !MCLR (portA, bit 5 (RA5))

;

; DISPLAY FREQUENCY OFFSET.  This feature adjusts the display frequency to take

; account of an intermediate frequency offset.  In this particular example, the

; IF frequency is above the DDS output and the actual frequency, so a subtract

; routine has been added.  Four minuend bytes define the IF frequency, and the

; subtract routine subtracts the calculated freq_0 through freq_3 bytes to

; yield four display bytes stored in display_0 through display_3.

;

;******************************************************************************

; Based on earlier work for AD9850 by Curtis W. Preuss - WB2V QEX 8/9/98

; Bruce Stough AA0ED 12/98 and 4/21/99

; Craig Johnson AA0ZZ 4/21/99 and others at AMQRP and

; NJQRP.  Many thanks to all prior trail blazers and authors!

;

; Modified and adapted to AD9834 DDS by David Harrison W6IBC who expressly

; dedicates any derivative and original works of authorship contained herein

; to the public domain.

;*****************************************************************************

; Revison History

;	8/11/04  Migrated code from RESET to this code including DDS reset sequence

;	and send_dds routine adapted to AD9834

;	8/12/04 Added routines to Calc DDS word and Send DDS for AD9834

;	Also theorized about ref_osc algorithm and adapted it to a 28 bit control

;	word.  Wonder if this is right?

;

;	8/16/04	Wrong.  The corrections are shown below. This code now works from

;	16F819 to AD9834DDS.  Have not yet reenabled the LCD code, because I'm

;	running PIC and DDS and XTAL at 3V dc (<10ma total).  In order to run

;	5 volt LCD either have to run PIC at 5 volts or step up control lines.

;	probably easier to run PIC at 5 volts and step down the three DDS control

;	lines.  Drawback:  Need two voltage regulators (+5 & +3.3) with this

;	approach. (Wrong)  Am running DDS, PIC and LCD at 5 volts

;	and xtal osc at 3 volts thru 160 ohm dropping resistor, 25 ma total draw.

;

;	8/27/04 Received PCBs yesterday, built up and restored LCD code.  DDS is

;	jumping around with the encoder, but display reads completely. Need to

;	check main line code to be sure LCD is not interfering with DDS process.

;	Modified LCD code to replace CLRF PORTB with MOVLW b'00000001' so that

;	Port B bit zero remains high except during DDS load sequence.  This is

; 	a critical difference between AD9850 and AD9834.

;

;	9-15-04 Restored IF offset (subtract), check add & subtract routines.

;	Also reordered program routines as follows:

;		Parameters, defines and equates

;		Start

;		Main

;		Encoder and step size

;		DDS including calculation of F and conversion to AD9834 format

;		Calibrate set for 10MHz for zero beat with WWV

;		LCD initialize and display

;		EEPROM Write, Read

;   	Timing Delays

;

; 	10-7-04	Reenabled Calibrate mode and EEPROM

;	10-10-04 Debugged Calibrate and EEPROM read and write for PIC16F819

;	10-23-04 Set up new project for NN1G NE602 Receiver with +4kHz offset

;		now Ver. 2.01

;   11-8-04  Rewrite DDS conversion to AD9834 routine, now Ver 3.00

;*****************************************************************************

;

; Target Controller -      PIC16F819 (same pinout as PIC16F628)

;                          __________

; PB1 ENCODER SWITCH--RA2 |1       18| RA1---------ENCODER A

;    PB2 SWITCH-------RA3 |2       17| RA0---------ENCODER B

;    PB4 SWITCH ------RA4 |3       16| OSC1--------XTAL

;    PB3 SWITCH-----!MCLR |4       15| OSC2--------XTAL

;     Ground----------Vss |5       14| VDD---------+5 V

;     DDS LOAD--------RB0 |6       13| RB7---------DDS DATA/LCD 14

;     LCD_rs----------RB1 |7       12| RB6---------LCD 13

;     LCD_rw----------RB2 |8       11| RB5---------DDS CLOCK/LCD 12

;     LCD_e-----------RB3 |9       10| RB4---------LCD 11

;                          ----------

;

; ****************************************************************************

; * Device type and options.                                                 *

; ****************************************************************************

;

list		p=PIC16F819

#include	<p16F819.inc>

radix     dec

errorlevel -302

;

; Configuration register settings

;

__CONFIG  H'3F29'

;	DBH note:  see PIC16F819 datasheet, page 90, for Config bit definitions

;	inter alia external crystal oscillator, and hard reset on RA5, no code

;   protect, no watch dog timer.

;

;

; ****************************************************************************

; Info for power-up display								     *

; ****************************************************************************

MCODE_REV_0	equ	'3'		; current version is 2.01 version

MCODE_REV_1 equ	'.'		; code migration from ver 7.05 DDS code

MCODE_REV_2 equ	'0'

MCODE_REV_3 equ	'0'

; ***************************************************************************

;  General equates.  These may be changed to accommodate the reference clock

;  frequency, the desired upper frequency limit, the desired lower frequency

;  limit, the subtrahend for IF offset and the default startup frequency.

; ****************************************************************************

;

; ref_osc represents the change in the frequency control word which results

; in a 1 Hz change in output frequency.  It is interpreted as a fixed point

; integer in the format <ref_osc_3>.<ref_osc_2><ref_osc_1><ref_osc_0>

;

; The values for common oscillator frequencies for AD9834 are as follows:

;

; Frequency    ref_osc_3    ref_osc_2    ref_osc_1    ref_osc_0

;

;  50.00 MHz     0x05         0x5E         0x63         0xB8

;  32.00 MHz     0x08		0x63		 0x7B		  0xD0

;  25.00 MHz     0x0A		0x6C		 0xC7		  0x71

;

;

;	The AD9834 uses a 28 bit control word rather than a 32 bit control

;	word as used in AD9850 for example.

;

;	Following the above algorithm in the same fixed point format

;	ref_osc_3 = (2^28/oscillator_freq_in_Hertz).

;	ref_osc_2, ref_osc_1 and Ref_osc_0 are the fractional part of

;	 (2^28/oscillator_freq_in_Hertz) times 2^24 in hex

;

;    Note: 	2^28 = 268,435,456

;			2^24 = 16777216

;

; For example, for a 50 MHz clock with AD9834 algorithm (2^28)

;    ref_osc_3 is (2^28 / 50 x 10^6) = 5.36870912 truncated to 5 (0x05)

;    ref_osc_2 is the high byte of (.36870912 x 2^24) = 618591.547

;    = 0x5E63B8, so fraction high byte is 5E.

;    ref_osc_1 is the next byte of 0x5E63B8, or 63

;    ref_osc_0 is the last byte of 0x5E63B8, or B8

;

;

; Currently set for 50 MHz Oscillator using AD9834 Algorithm

ref_osc_3   equ 0x05            ; Most significant osc byte

ref_osc_2   equ 0x5E            ; Next byte

ref_osc_1   equ 0x63            ; Next byte

ref_osc_0   equ 0xB8            ; Least significant byte

;

; Maximum Frequency Limit

; Limit contains the upper limit frequency as a 32 bit integer.

; This should not be set to more than one third of the reference

; oscillator frequency.  The output filter of the DDS board must

; be designed to pass frequencies up to the maximum.

; THIS VALUE HAS BEEN CHANGED FROM

; 30 MHZ 01 C9 C3 80 TO 5.0 MHZ 00 4C 5A E0

;

limit_3   equ 0x00              ; Most significant byte for 5.004 MHz

limit_2   equ 0x4C              ; Next byte

limit_1   equ 0x5A              ; Next byte

limit_0   equ 0xE0              ; Least significant byte

;

; Minimum Frequency Limit

; Lowest contains the lower limit cutoff frequency as a 32 bit integer.

;	In this application lowest frequency is 4.704 MHz = 47 B7 60

lowest_3	equ	0x00		  ; Most significant byte for 4.704 MHz

lowest_2	equ 0x47

lowest_1	equ 0xC7

lowest_0	equ 0x00

;

; STARTUP Frequency

; Default contains the default startup frequency as a 32 bit BINARY integer,

;  expressed as eight hex values NOTE:  This value is raw hex, not

;	converted to the final 28 bit data word which is:

;	LS word 72 9B and MS word 46 15.  If you change STARTUP freq here, you

;   must also change the initialize DDS in the START routine which uses

;	the actual converted values most easily obtained from Analog Devices

;   on-line tool for the AD9834.

startup_3 equ 0x00             ; Most significant byte for 4.754 MHz

startup_2 equ 0x48             ; Next byte

startup_1 equ 0x8A             ; Next byte

startup_0 equ 0x50             ; Least significant byte

;

;

; IF Offset

; The Receiver IF uses 12MHz crystals with local carrier at 12.004 MHz

; These following constants define an IF frequency offset of 12,004,000 Hz.

; Subtract subroutine subtracts the calculated frequency bytes from these

; four minuends to produce four IF-corrected DISPLAY bytes.  To change an

; IF frequency ABOVE actual receive freqency, change these four bytes.

;

minuend_3 equ 0x00		; Most significant minuend byte

minuend_2 equ 0xB7		; Next minuend byte

minuend_1 equ 0x2A		; Next minuend byte

minuend_0 equ 0xA0		; Least significant minuend byte

;

;

;*************************************************************

;  Setup the initial constant, based on the frequency of the reference

;  oscillator.  This can be corrected with the calibrate function.

; ***********************************************************************

;

ORG     0x2100		; MPASM addresses EEPROM beginning at 0x2100

DE    ref_osc_0

DE    ref_osc_1

DE    ref_osc_2

DE    ref_osc_3

;

;       Clear unused EEPROM bytes.

;

DE    0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0

DE    0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0,0

;

;

; **************************************************************************

;  Bit numbers for the STATUS file register:

; **************************************************************************

;

B_RP1	  EQU	    6

B_RP0   EQU     5

B_NTO   EQU     4

B_NPD   EQU     3

B_Z     EQU     2

B_DC    EQU     1

B_C     EQU     0

;

; **********************************************************************

;  Assign names to IO pins.

; **********************************************************************

;

;   B register bits:

;

DDS_fsync equ   0x00          ; Update pin on AD9834

LCD_rs  equ     0x01          ; 0=instruction, 1=data

LCD_rw  equ     0x02          ; 0=write, 1=read

LCD_e   equ     0x03          ; 0=disable, 1=enable

DDS_clk equ     0x05          ; AD9834 write clock

DDS_dat equ     0x07          ; AD9834 serial data input

;

;   A register bits:

pb_4 equ   0x04			; wired to PortA, bit 4 (not used)

pb_2 equ   0x03              	; calibrate pin on header (RA3)

pb_1 equ   0x02			; step switch on encoder pushbutton

;	note: In 16F819 code pb3 is hardwired to reset pin port A,

;		pin 5 (_MCLRE_ON in CONFIG)

;

;***********************************************************************

; Define Special Register Banks 0, 1, 2 and 3 in PIC16F819

;***********************************************************************

;

#define 	Bank0	0x00

#define	Bank1	0x80

#define	Bank2	0x100

#define	Bank3	0x180

;

;************************************************************************

; Allocate variables in general purpose register space

;************************************************************************

;

CBLOCK  0x20            ; Starting pointer for register space

freq_0                   ; Display frequency (hex)

freq_1                  ;  (4 bytes)

freq_2

freq_3

BCD_0                    ; Display frequency (BCD)

BCD_1                   ;  (5 bytes)

BCD_2

BCD_3

BCD_4

AD9834_0                 ; AD9834 control word

AD9834_1                ;  (5 bytes)

AD9834_2			; (This perpetuates the AD9850

AD9834_3			; algorithm, only 4 bytes are needed

AD9834_4			; for AD9834 because of its freq range)

fstep_0                  ; Frequency increment/decrement

fstep_1                 ;  (4 bytes)

fstep_2

fstep_3

BCD_count                ; Used in bin2BCD routine

BCD_temp                 ;   "

mult_count               ; Used in calc_dds_word

bit_count                ;   "

byte2send                ;

osc_0                    ; Current oscillator

osc_1                   ;  (4 bytes)

osc_2

osc_3

osc_temp_0               ; Oscillator frequency

osc_temp_1              ;  (4 bytes)

osc_temp_2

osc_temp_3

LCD_char                 ; Character being sent to the LCD

LCD_read                 ; Character read from the LCD

timer1                   ; Used in delay routines

timer2                   ;   "

step_state               ; step size state register

ren_new                  ; New value of encoder pins A and B

ren_old                 ; Old value of encoder pins A and B

ren_read                ; Encoder pins A and B and switch pin

last_dir                ; Indicates last direction of encoder

next_dir                 ; Indicates expected direction

count                   ; loop counter  (gets reused)

rs_value                 ; The LCD rs line flag value

display_0			; IF Offset Display least significant byte

display_1			; IF offset next display byte

display_2			; IF offset next display byte

display_3			; IF offset display high byte

dds_0				; AD9834-adjusted control word, low byte

dds_1				; (4 bytes)

dds_2

dds_3

send0				; send low byte  in DDS_send routine

send1				; send high byte in DDS_send routine

ENDC                    ; End of Data Block

;

; ***********************************************************************

;  The 16F819 resets to 0x00.

;  The Interrupt vector is at 0x04. (Unused)

; ***********************************************************************

;

ORG     0x0000

reset_entry

goto    start             ; Jump around interrupt vector at 0x0004

;

;

;

;*************************************************************************

; start routine

;*************************************************************************

;

start

; Port A configuration

clrf	  INTCON

clrf	  PORTA	      ; Initialize Port A by setting output data

; latches

bcf	  STATUS,RP1

bsf	  STATUS,B_RP0    ; select Bank 1

movlw	  0x06	      ; Turn off comparators (set lower 3 bits)

movwf	  ADCON1	      ; of ADCON1  for I/O on pins 2, 1 and 0

movlw	  0xFF	      ; Port A pins set as inputs

movwf	  TRISA	      ; Note TRISA<5> is always an input, read as 1

; Port B Configuration

clrf	  OPTION_REG

clrf    TRISB	      ; Set all port B pins as outputs

bcf     STATUS,B_RP0    ; Switch back to bank 0

movlw	  0x01

movwf	  PORTB

; Initialize LCD and display version

call	  init_LCD

call	  display_version

; Entry point for calibrate routine

btfsc	  PORTA,pb_2      ; is the calibrate switch pressed?

goto	  read_Eeocs      ; no, get clock frequency from EEPROM

call	  calibrate	      ; yes, go to oscillator calibrate routine

;  Get the reference oscillator constant from the EEPROM.

;

read_Eeocs

banksel  Bank2	      ; switch to bank 2

clrf     EEADR           ; Reset the EEPROM read address

call     read_EEPROM     ; Read EEPROM

movwf    osc_0           ; Save osc frequency

call     read_EEPROM     ; Get next byte

movwf    osc_1           ; Save it

call     read_EEPROM     ; Get the third byte

movwf    osc_2           ; Save it

call     read_EEPROM     ; Get the fourth byte

movwf    osc_3           ; Save it

;

;  Set the power on frequency to the defined value.

;

movlw    startup_0       ; Get the least significant byte

movwf    freq_0          ; Save it

movlw    startup_1 	; Get the next byte

movwf    freq_1    	; Save it

movlw    startup_2 	; And the next

movwf    freq_2    	; Save it

movlw    startup_3 	; Get the most significant byte

movwf    freq_3    	; Save it

;

;       Display the power on frequency.

;

call	  subtract		; make IF offset adjustment to pwr-on freq

call     bin2BCD         ; Convert it to BCD

call     show_freq       ; Display it

;

;		Initialize DDS	NOTE THESE AD9834-MODIFIED CONTROL WORDS

; 		 			SHOULD MATCH THOSE FOR STARTUP FREQ.

movlw	  0x72

movwf	  AD9834_1		; low word MSB

movlw	  0x9B

movwf	  AD9834_0		; low word LSB

movlw	  0x46

movwf	  AD9834_3		; high word MSB

movlw	  0x15

movwf	  AD9834_2		; high word LSB

call	  reset_DDS

;       Get the power on encoder value.

;

movf     PORTA,w          ; Read port

movwf    ren_read         ; Save it in ren_read

movlw    0x03             ; Get encoder mask

andwf    ren_read,w       ; Get encoder bits

movwf    ren_old          ; Save in ren_old

;

;       Initialize encoder variables.

;

clrf     last_dir          ; Set the knob direction indicator to Down

movlw	  0x01		  ; Initially set step-size to 100Hz step

movwf	  step_state

call	  chg_step		  ; advances to 1Khz step size

;

; Fall into the Main Program Loop

;

; ****************************************************************************

; * main routine  (DBH rewrite)

; * Purpose:  This is the Main Program Loop. The program's main loop

; *           calls poll_encoder. Poll_encoder continuously polls the rotary

;		  shaft encoder. When the shaft encoder has changed, the direction

; *           it moved is determined and stored in last_dir.  The subroutine

; *           then returns to main.

; *

; *		  If pb_1 is pressed, step size cycles through 1kHz, 10kHz, 10Hz

; *           and 100Hz.  Startup default is 1KHz   The variable

; *           fstep is added or subtracted from the current VFO frequency

; *           stored in freq. The contents of freq are then converted to a

; *           BCD number in subroutine bin2BCD.  The subroutine show_freq is

; *           then called to display the result on the Liquid Crystal Display.

; *           Next, the subroutine calc_dds_word is used to calculate the DDS

; *           frequency control word from the values in freq and osc.

; *           The result is stored in AD9834.  This data is transferred to

; *           the AD9834 DDS chip by calling the subroutine send_dds_word.

; *

; *   Input:  None.

; *

; *  Output:  None.

; *

; *****************************************************************************

;

main

call	poll_encoder	;  poll encoder loop

;

; now make the freq change

;

btfsc  last_dir,1  	; Is the knob going up?

goto   up                ; yes, add fstep increment to f

down

call   sub_step          ; no, subtract fstep from freq

call	check_sub		; make sure did not exceed minimum f

goto	update

up

call	add_step		; Yes, add fstep to freq

call 	check_add		; limit f to max freq

update

call	subtract		; Make IF offset adjustment for display only

call   bin2BCD           ; Convert the adjusted display frequency to BCD

call   show_freq         ; Display the frequency on the LCD

call   calc_dds_word     ; Find the control word for the DDS chip

call   send_dds_seq      ; Send the control word to the DDS chip

goto   main              ; Continue main loop

;

;

; *****************************************************************************

; * poll_encoder routine  Program basically loops here until encoder changes

; * Purpose:  Polls quadrature encoder:

; *             Reads the encoder bits until a change is detected, then

; *                 determines the direction the knob was moved.              *

; *                                                                           *

; *   Input:  Knob input read from port A                                     *

; *           ren_old -> the last encoder bits read                           *

; *           last_dir -> the last direction moved                            *

; *                                                                           *

; *  Output:  ren_timer -> an indication the speed of the knob.               *

; *           ren_new -> the current encoder bits                             *

; *           last_dir -> the last direction (0 = down, 2 = up)               *

; *                                                                           *

; *****************************************************************************

;

poll_encoder

bcf	STATUS,B_C

movf	PORTA,w		; Read port A incl encoder bits

btfss	PORTA,pb_1	  	; Is PB-1 presssed?

call	chg_step		; yes, change step size

bcf	STATUS,B_C

movf	PORTA,w

movwf	ren_read

movlw  0x03              ; no, set encoder mask

andwf  ren_read,w        ; and isolate encoder bits

movwf  ren_new           ; Save new value of encoder bits

xorwf  ren_old,w         ; Has 2-bit pattern changed?

btfsc  STATUS,B_Z        ; If zero flag is not set, no change

goto	poll_encoder	; so, no change, continue to poll

goto	check_direction	;	Yes, go to check direction

;

;**********************************************************************

; check_direction (encoder_chg) routine

;

;  Encoder bits are on RA0 and RA1 - the two low order bits of ren_new

;   A and B are "gray code" - 90 degrees out of phase (quadrature)

;         ___     ___

;        |   |   |   |

; A  ____|   |___|   |___

;           ___     ___

;          |   |   |   |

; B     ___|   |___|   |___

;       ^ ^ ^ ^ ^ ^ ^ ^

;       a b c d a b c d

;

;              A B

; At point a:  0 0

; At point b:  1 0

; At point c:  1 1

; At point d:  0 1

;

; Going UP, the sequence is a,b,c,d,a,b,c,d, etc. so the sequence is:

;     00, 10, 11, 01, 00, 10, 11, 01, etc.

;

; Going DOWN, the sequence is d,c,b,a,d,c,b,a, etc. so the sequence is:

;     01, 11, 10, 00, 01, 11, 10, 00, etc.

;

; To determine if the sequence is UP or DOWN:

;   1) Take the "Right-Bit" of any pair.

;   2) XOR it with the "Left-Bit" of the next pair in the sequence.

;   3) If the result is 1 it is UP

;      If the result is 0 it is DOWN

;

; The direction flag is 0 (DOWN) or 2 (UP) because of bit positioning

;**************************************************************************

;

check_direction

bcf	STATUS,B_C	  	  ; clear the carry flag before rotating

rlf	ren_old,f	  	  ; rotate old enc bits left

movf	ren_new,w		  ; move ren_new to w

xorwf	ren_old,f		  ; xor w with ren_old, result in w

movf	ren_old,w		  ; move ren_old to w

andlw	0x02			  ; AND w with 02Hex to select bit 1

movwf	next_dir		  ; store result in next_dir

xorwf	last_dir,w		  ; XOR w with last_dir, store in w

;

;	prevent encoder slip from giving a false change in direction

btfsc	STATUS,B_Z		  ; xero flag set? (is direction same?)

goto	pe_continue		  ; Yes, same direction, so no slip

movf 	next_dir,w		  ; No, direction changed, so update

movwf	last_dir		  ; Update direction indicator

movf	ren_new,w		  ; move current encoder bits from w

movwf	ren_old		  ; to ren_old for next time

return

pe_continue

clrf	last_dir		  ; Clear last direction (default=dn)

btfss	ren_old,1		  ; Are we going up?

goto	exit3			  ; No, finish up

up2

movlw	0x02			  ; Get UP constant

movwf	last_dir		  ; and put in last_dir

exit3

movf	ren_new,w		  ; Get current encoder bits

movwf	ren_old			  ; Save in ren_old for next time

return

;

;*********************************************************************

;

; Change Step routine

;

;	called from Main so that when invoked, the frequency does

;	not change

;

;*********************************************************************

;

chg_step

movlw	0xC0		; set LCD cursor at 2nd line, pos 8

movwf	LCD_char

call	cmnd2LCD	; and write "STEP = "

movlw	'S'

movwf	LCD_char

call	data2LCD

movlw	'T'

movwf	LCD_char

call	data2LCD

movlw	'E'

movwf	LCD_char

call	data2LCD

movlw	'P'

movwf	LCD_char

call	data2LCD

movlw	' '

movwf	LCD_char

call	data2LCD

movlw	'='

movwf	LCD_char

call	data2LCD

movlw	' '

movwf	LCD_char

call	data2LCD

; 	Implements an fstep state machine for computed goto

incf	step_state,f

movf	step_state,w

andlw	B'00000011'

addwf	PCL,f

goto	set_step_10h

goto	set_step_100h

goto	set_step_1k

goto	set_step_10k

;

set_step_10h		; set step size to 10 Hz

clrf	fstep_3

clrf	fstep_2

clrf	fstep_1

movlw	0x0A

movwf	fstep_0

;				display "10 Hz"

movlw	0xC7

movwf	LCD_char

call	cmnd2LCD

movlw	'1'

movwf	LCD_char

call	data2LCD

movlw	'0'

movwf	LCD_char

call	data2LCD

movlw	' '

movwf	LCD_char

call	data2LCD

movlw	'H'

movwf	LCD_char

call	data2LCD

movlw	'z'

call	data2LCD

movlw	' '

movwf	LCD_char

call	data2LCD

call	wait_256ms

return

;

set_step_100h		; set step size to 100 Hz

clrf	fstep_3

clrf	fstep_2

clrf	fstep_1

movlw	0x64

movwf	fstep_0

;				display 100 Hz

movlw	0xC7

movwf	LCD_char

call	cmnd2LCD

movlw	'1'

movwf	LCD_char

call	data2LCD

movlw	'0'

movwf	LCD_char

call	data2LCD

movlw	'0'

movwf	LCD_char

call	data2LCD

movlw	' '

movwf	LCD_char

call	data2LCD

movlw	'H'

movwf	LCD_char

call	data2LCD

movlw	'z'

movwf	LCD_char

call	data2LCD

call	wait_256ms

return

;

set_step_1k			; set step size to 1 KHz

clrf	fstep_3

clrf	fstep_2

movlw 	0x03

movwf	fstep_1

movlw	0xE8

movwf	fstep_0

;				display 1 KHz

movlw	0xC7

movwf	LCD_char

call	cmnd2LCD

movlw	'1'

movwf	LCD_char

call	data2LCD

movlw	' '

movwf	LCD_char

call	data2LCD

movlw	'k'

movwf	LCD_char

call	data2LCD

movlw	'H'

movwf	LCD_char

call	data2LCD

movlw	'z'

movwf	LCD_char

call	data2LCD

movlw	' '

movwf	LCD_char

call	data2LCD

movlw	' '

movwf	LCD_char

call	data2LCD

call	wait_256ms

return

;

set_step_10k		; set step size to 10 kHz

clrf	fstep_3

clrf	fstep_2

movlw 	0x27

movwf	fstep_1

movlw	0x10

movwf	fstep_0

; 				display "10 kHz"

movlw	0xC7

movwf	LCD_char

call	cmnd2LCD

movlw	'1'

movwf	LCD_char

call	data2LCD

movlw	'0'

movwf	LCD_char

call	data2LCD

movlw	' '

movwf	LCD_char

call	data2LCD

movlw	'k'

movwf	LCD_char

call	data2LCD

movlw	'H'

movwf	LCD_char

call	data2LCD

movlw	'z'

movwf	LCD_char

call	data2LCD

movlw	' '

movwf	LCD_char

call	data2LCD

movlw	' '

movwf	LCD_char

call	data2LCD

call	wait_256ms

return

;

;***************************************************************************

; add_step routine

; Purpose:  This routine adds the 32 bit value of fstep to the 32 bit

;           value in freq.  When incrementing, the fstep value is a

;           positive integer.  When decrementing, fstep is the complement

;           of the value being subtracted.

;

;   Input:  The 32 bit values in fstep and freq

;

;  Output:  The sum of fstep and freq is stored in freq.  When incrementing

;           this value may exceed the maximum.  When decrementing, it may

;           go negative.

;

;**************************************************************************

add_step

movf    fstep_0,w         ; Get low byte of the increment

addwf   freq_0,f          ; Add it to the low byte of freq

btfss   STATUS,B_C        ; Any carry?

goto    add1              ; No, add next byte

incfsz  freq_1,f          ; Ripple carry up to the next byte

goto    add1              ; No new carry, add next byte

incfsz  freq_2,f          ; Ripple carry up to the next byte

goto    add1              ; No new carry, add next byte

incf    freq_3,f          ; Ripple carry up to the highest byte

add1

movf    fstep_1,w         ; Get the next increment byte

addwf   freq_1,f          ; Add it to the next higher byte

btfss   STATUS,B_C        ; Any carry?

goto    add2              ; No, add next byte

incfsz  freq_2,f          ; Ripple carry up to the next byte

goto    add2              ; No new carry, add next byte

incf    freq_3,f          ; Ripple carry up to the highest byte

add2

movf    fstep_2,w         ; Get the next to most significant increment

addwf   freq_2,f          ; Add it to the freq byte

btfss   STATUS,B_C        ; Any carry?

goto    add3              ; No, add last byte

incf    freq_3,f          ; Ripple carry up to the highest byte

add3

movf    fstep_3,w         ; Get the most significant increment byte

addwf   freq_3,f          ; Add it to the most significant freq

return                    ; Return to the caller

;

;*************************************************************************

; check_add routine

; Purpose:  Check if freq exceeds the upper limit.

;

;   Input:  The 32 bit values in freq

;

;  Output:  If freq is below the limit, it is unchanged.  Otherwise, it is

;           set to equal the upper limit.

;

; *************************************************************************

;

check_add

;

;       Check the most significant byte.

;

movlw   0xFF-limit_3      ; Get (FF - limit of high byte)

addwf   freq_3,w          ; Add it to the current high byte

btfsc   STATUS,B_C        ; Was high byte too large?

goto    set_max           ; Yes, apply limit

movlw   limit_3           ; Get high limit value

subwf   freq_3,w          ; Subtract the limit value

btfss   STATUS,B_C        ; Are we at the limit for the byte?

goto    exit1             ; No, below.  Checks are done.

;

;       Check the second most significant byte.

;

movlw   0xFF-limit_2      ; Get (FF - limit of next byte)

addwf   freq_2,w          ; Add it to the current byte

btfsc   STATUS,B_C        ; Is the current value too high?

goto    set_max           ; Yes, apply the limit

movlw   limit_2           ; Second limit byte

subwf   freq_2,w          ; Subtract limit value

btfss   STATUS,B_C        ; Are we at the limit for the byte?

goto    exit1             ; No, below.  Checks are done.

;

;       Check the third most significant byte.

;

movlw   0xFF-limit_1      ; Get (FF - limit of next byte)

addwf   freq_1,w          ; Add it to the current byte

btfsc   STATUS,B_C        ; Is the current value too high?

goto    set_max           ; Yes, apply the limit

movlw   limit_1           ; Third limit byte

subwf   freq_1,w          ; Subtract limit value

btfss   STATUS,B_C        ; Are we at the limit for the byte?

goto    exit1             ; No, below.  Checks are done.

;

;       Check the least significant byte.

;

movlw   limit_0           ; Fourth limit byte

subwf   freq_0,w          ; Subtract limit value

btfss   STATUS,B_C        ; Are we at the limit for the byte?

goto    exit1             ; No, below.  Checks are done.

set_max

movlw   limit_0           ; Get least significant limit

movwf   freq_0            ; Set it in freq

movlw   limit_1           ; Get the next byte limit

movwf   freq_1            ; Set it in freq_1

movlw   limit_2           ; Get the next byte limit

movwf   freq_2            ; Set it in freq_2

movlw   limit_3           ; Get the most significant limit

movwf   freq_3            ; Set it in freq_3

exit1

return                    ; Return to the caller

;

; *****************************************************************************

; * sub_step routine                                                          *

; * Purpose:  Subtract the increment step from freq, checking that it does    *

; *           not go below zero.                                              *

; *                                                                           *

; *   Input:  The values in fstep and freq.                                   *

; *                                                                           *

; *  Output:  The updated value in freq.                                      *

; *                                                                           *

; *****************************************************************************

;

sub_step

;

;	new subtraction routine added by DBH 7-4-04

movf	fstep_0,w

subwf	freq_0,f

movf	freq_1,w

btfss	STATUS,B_C

addlw	0xFF

movwf	freq_1

movf	fstep_1,w

subwf	freq_1,f

movf	freq_2,w

btfss	STATUS,B_C

addlw	0xFF

movwf	freq_2

movf	fstep_2,w

subwf	freq_2,f

movf	fstep_3,w

subwf	freq_3,f

return

;

;*********************************************************************************

;		Check_sub routine

;

;		Check Subtraction step to see if frequency has gone below the lower limit

;		If the frequency has gone below the lower limit, set it to the lower

;		limit.

;		The lower limit values are literals lowest_3-0

;		The MSB lowest literal (lowest_3) is loaded into w reg.

;		W reg is then subtracted from the MSB of current frequency (freq_3)

;

;		If carry =1 and zero =0, the result is positive

;		and we can exit this check routine.  If carry and zero =1, the result is

; 		zero, so the next lower byte must be checked. All bytes are progressively

;		checked until it is established that the new freq is above or below

;		the minimum frequency limit.

; 		At any time that carry flag goes false, the minimum freq is imposed.

;

; 		NEWLY ADDED BY DBH 6-18-04

;*********************************************************************************

;       Check the most significant byte.

;

check_sub

movlw   lowest_3          ; move lowest_3 to w

subwf   freq_3,w          ; subtr w from literal lowest_3

btfss   STATUS,B_C        ; if carry flag is zero then

goto    set_min           ; apply lower limit

btfss	STATUS,B_Z	      ; is the zero flag set

goto    exit2    		; no, value is greater than lower limit

;       Check the second most significant byte.

movlw   lowest_2	      ; yes, check next byte

subwf   freq_2,w          ;

btfss   STATUS,B_C        ; Is the current value too high?

goto    set_min           ; Yes, apply the limit

btfss   STATUS,B_Z        ; Is zero flag set?

goto    exit2             ; No, above.  Checks are done.

;       Check the third most significant byte.

movlw	lowest_1      	; yes, check next byte

subwf   freq_1,w          ;

btfss   STATUS,B_C        ; Is the current value too high?

goto    set_min           ; Yes, apply the limit

btfss   STATUS,B_Z        ; Is zero flag set?

goto    exit2             ; No, above.  Checks are done.

;       Check the least significant byte.

movlw   lowest_0    	  ; yes, check next byte

subwf   freq_0,w          ;

btfss   STATUS,B_C        ; Are we at the limit for the byte?

goto	set_min

btfss	STATUS,B_Z

goto    exit2             ; No, above.  Checks are done.

set_min

movlw   lowest_0          ; Get least significant limit

movwf   freq_0            ; Set it in freq

movlw   lowest_1          ; Get the next byte limit

movwf   freq_1            ; Set it in freq_1

movlw   lowest_2          ; Get the next byte limit

movwf   freq_2            ; Set it in freq_2

movlw   lowest_3          ; Get the most significant limit

movwf   freq_3            ; Set it in freq_3

exit2

return                    ; Return to the caller

;

;**************************************************************************

;	reset_DDS

;

;	Function:	Reset AD9834 DDS chip with required sequence per data sheet

;	The data sheet requires a control word of 0x2100, the low freq word

;	(2 control bits and 14 LSBs), the high freq word (2 control bits and 14

;   MSBs), 0x8000 to clear FREG 1 LSB, 0x8000 to clear Freg1 MSB, 0xC000 to

;	clear phase 0 reg, 0xE000 to clear phase 1 reg, and 0x2000 to clear

;   reset state and enable DDS output.

;

;**************************************************************************

reset_DDS

movlw		0x21			; this is a control word that sets reset

movwf		send1			; and selects FReg0

clrf		send0

call		send_dds

;		The following values were determined using Analog Devices' online

;		"Interactive Design Tools, DDS, AD9834 Device Configuration Assistant

;	  but can be calculated using algorithm above for AD9834

movf		AD9834_1,w		; 14 LSBs of starting frequency in Reg 0

movwf		send1			; starting freq is 4.754 MHz

movf		AD9834_0,w

movwf		send0

call		send_dds

movf		AD9834_3,w		; 14 MSBs of starting freq in Reg 0

movwf		send1

movf		AD9834_2,w

movwf		send0

call		send_dds

movlw		0x80			; clears FReg 1 LSB

movwf		send1

clrf		send0

call		send_dds

movlw		0x80			; clears FReg 1 MSB

movwf		send1

clrf		send0

call		send_dds

movlw		0xC0			; clears phase 0 reg

movwf		send1

clrf		send0

call		send_dds

movlw		0xE0			; clears phase 1 reg

movwf		send1

clrf		send0

call		send_dds

movlw		0x20			; Control word, clears reset

movwf		send1			; enabling DDS output of

clrf		send0			; starting frequency

call		send_dds

return

;

; *****************************************************************************

; * calc_dds_word routine

; * Purpose:  Multiply the 32 bit number for oscillator frequency times the

; *           32 bit number for the displayed frequency.

; *           (unchanged from the AD9850 code, except as noted below)

; *

; *   Input:  The reference oscillator value in osc_3 ... osc_0 and the

; *           current frequency stored in freq_3 ... freq_0.  The reference

; *           oscillator value is treated as a fixed point real, with a 24

; *           bit mantissa.

; *

; *  Output:  The result is stored in AD9834_3 ... AD9834_0.

; *

; *****************************************************************************

;

calc_dds_word

clrf    AD9834_0          ; Clear the AD9834 control word bytes

clrf    AD9834_1          ;

clrf    AD9834_2          ;

clrf    AD9834_3          ;

clrf    AD9834_4          ;

movlw   0x20              ; Set count  to 32   (4 osc bytes of 8 bits)

movwf   mult_count        ; Keep running count

movf    osc_0,w           ; Move the four osc bytes

movwf   osc_temp_0        ; to temporary storage for this calculation

movf    osc_1,w           ; (Don't disturb original osc bytes)

movwf   osc_temp_1        ;

movf    osc_2,w           ;

movwf   osc_temp_2        ;

movf    osc_3,w           ;

movwf   osc_temp_3        ;

mult_loop

bcf     STATUS,B_C        ; Start with Carry clear

btfss   osc_temp_0,0      ; Is bit 0 (Least Significant bit) set?

goto    noAdd             ; No, don't need to add freq term to total

movf    freq_0,w          ; Yes, get the freq_0 term

addwf   AD9834_1,f        ;   and add it in to total

btfss   STATUS,B_C        ; Does this addition result in a carry?

goto    add7              ; No, continue with next freq term

incfsz  AD9834_2,f        ; Yes, add one and check for another carry

goto    add7              ; No, continue with next freq term

incfsz  AD9834_3,f        ; Yes, add one and check for another carry

goto    add7              ; No, continue with next freq term

incf    AD9834_4,f        ; Yes, add one and continue

add7

movf    freq_1,w          ; Use the freq_1 term

addwf   AD9834_2,f        ; Add freq term to total in correct position

btfss   STATUS,B_C        ; Does this addition result in a carry?

goto    add8              ; No, continue with next freq term

incfsz  AD9834_3,f        ; Yes, add one and check for another carry

goto    add8              ; No, continue with next freq term

incf    AD9834_4,f        ; Yes, add one and continue

add8

movf    freq_2,w          ; Use the freq_2 term

addwf   AD9834_3,f        ; Add freq term to total in correct position

btfss   STATUS,B_C        ; Does this addition result in a carry?

goto    add9              ; No, continue with next freq term

incf    AD9834_4,f        ; Yes, add one and continue

add9

movf    freq_3,w          ; Use the freq_3 term

addwf   AD9834_4,f        ; Add freq term to total in correct position

noAdd

rrf     AD9834_4,f        ; Shift next multiplier bit into position

rrf     AD9834_3,f        ; Rotate bits to right from byte to byte

rrf     AD9834_2,f        ;

rrf     AD9834_1,f        ;

rrf     AD9834_0,f        ;

rrf     osc_temp_3,f      ; Shift next multiplicand bit into position

rrf     osc_temp_2,f      ; Rotate bits to right from byte to byte

rrf     osc_temp_1,f      ;

rrf     osc_temp_0,f      ;

decfsz  mult_count,f      ; One more bit has been done.  Are we done?

goto    mult_loop         ; No, go back to use this bit

clrf    AD9834_4          ; Yes, clear _4.  Answer is in bytes _3 .. _0

;

;*************************************************************************

;	AD9834 Modification to original AD9850 code:

;	Newly added sequence to create a 28 bit control word comprising two

;	14 bit data words and a two bit register select preamble, results in

;	four bytes dds_3, dds_2, dds_1 and dds_0.

;     (see AD9834 data sheet, Table 7).

;**************************************************************************

;

;	Handle low order byte of LSB word.

movf	AD9834_0,w		; this lowest order byte is unmodified,

movwf	dds_0			; and can be used directly.

;	Modify high order byte of LSB word by adding 2-bit preamble.

movf	AD9834_1,w

movwf	osc_temp_1		; using osc_temp_1 for temp storage

;	Add two control bits to high byte of LS word.

bsf	osc_temp_1,6	; force bit 6 high

bcf	osc_temp_1,7	; and bit 7 low, sel FReg0 of AD9834

movf	osc_temp_1,w

movwf	dds_1			; modified high byte of LS word now in dds_1

;	Handle high order byte of MSB word by left shifting the three

;   high order bytes in AD9834_1, AD9834_2 and AD9834_3 through

;	the carry bit.

movf	AD9834_1,w		; get unmodified value

movwf	osc_temp_1		; store it back into osc_temp_1

movf	AD9834_2,w		; get the next unmodified value

movwf	osc_temp_2		; store in osc_temp_2

movf	AD9834_3,w		; get the high byte unmodified value

movwf	osc_temp_3		; store in osc_temp_3

;	Now we're ready to rotate these three bytes two positions to the

;	left to make up for 2-bit control values of dds_1.

bcf	STATUS,B_C        ; clear carry flag of STATUS register

rlf	osc_temp_1,f	; rotate the three high order bytes two

rlf	osc_temp_2,f	; positions left, through carry flag of

rlf	osc_temp_3,f	; STATUS register.

rlf	osc_temp_1,f

rlf	osc_temp_2,f

rlf	osc_temp_3,f

bcf	STATUS,B_C

;	Since we've moved the two high order bits from original AD9834_1

;	to the two low bit positions of AD9834_2, we no longer need the

;	osc_temp_1 value, so it can be ignored.

movf	osc_temp_2,w

movwf	dds_2

;	Now modify the high byte of the MS control word

bsf		osc_temp_3,6	; force high bit 6 to be 1 and

bcf		osc_temp_3,7	; high bit 7 to be zero, sel FReg0

movf	osc_temp_3,w

movwf	dds_3

;	End of new code adaptation for AD9834

return

;

;*************************************************************************

;

;	send-dds-seq		Unique for AD9834

;	Purpose:  Create a sequence comprising a 16 bit control word

;			followed by:

;			a 16 bit data word including two register select MS bits

;			and 14 LSB data bits, followed by

;

;			a 16 bit data word including two register select MS bits

;			and 14 MSB data bits.  See Table VII of AD9834 data sheet

;	Input:	dds_3 . . . dds_0

;	Output:  sequence of send1, send0 16 bit sequences to send_dds

;

;**************************************************************************

;

send_dds_seq		; Unique for AD9834

;

movlw	0x20		; This is the 16 bit control word for FReg 0

movwf	send1		; High nibble 2 sets up three write sequences

movlw	0x00

movwf	send0

call	send_dds

;

movf	dds_1,w	; Low order frequency word incl 14 LSBs

movwf	send1		; and 2-bit preamble

movf	dds_0,w

movwf	send0

call	send_dds

;

movf	dds_3,w	; High order frequency word incl 14 MSBs

movwf	send1		; and 2-bit preamble

movf	dds_2,w

movwf	send0

call	send_dds

;

return

;

; ************************************************************************

;  send_dds routine

;   Purpose:  This routine sends the AD9834 control

;			  word to the DDS chip

;			  using a serial data transfer.                               ;

;   Input:  send1 ... send0

;

;    Output:  The DDS chip register is updated.

;                                                                           **************************************************************************

;

send_dds

movlw   16              	; load up 16 bit count

movwf   count           	;

bsf	PORTB,DDS_clk	; raise DDS serial clock pin

bcf	PORTB,DDS_fsync	; lower DDS sync pin

send_loop

bcf	PORTB, DDS_dat	; assume current bit is a zero

btfsc	send1,7		; check current MSB of 16-bit word

bsf	PORTB,DDS_dat 	; wrong assumption, bit is a one

bcf	PORTB,DDS_clk	; trigger DDS serial clock pin

bsf	PORTB,DDS_clk	; finish clock pulse

bcf	PORTB,DDS_dat

rlf	send0,f		; rotate send0 contents one bit left

; through carry

rlf	send1,f		; rotate send1 contents through carry

decfsz count			; decrement the loop count

goto	send_loop		; not zero, continue to send

bsf	PORTB,DDS_fsync	; raise sync pin

return					; return to calling routine

;

;

;****************************************************************************

;	calibrate routine

;	Purpose:  Correct oscillator frequency. Routine is entered if pb_2 is

;	pressed or jumpered to ground at power on. DDS chip is programmed

;	to generate 10 MHz, based on osc value stored in EEPROM.  So long as

;	pb_2 is low, the osc value may be slowly altered to allow zero beat

;	with 10MHz standard, such as WWV at 10MHz.  Once pb-2 is taken high

;	followed by rotation of the encoder, a corrected osc value is

;	written into EEPROM, and the normal start-up sequence resumes.

;

;*****************************************************************************

;

calibrate

;	set up adjusted starting bytes for AD9834

;	AD9834 corrected bytes for 10 MHz are 0x4CCC7333

movlw	0x73

movwf	AD9834_1

movlw	0x33

movwf	AD9834_0

movlw	0x4C

movwf AD9834_3

movlw 0xCC

movwf	AD9834_2

;

;	now set uncorrected frequency display bytes to 10 MHz

movlw	0x80

movwf	freq_0

movwf	display_0

movlw	0x96

movwf	freq_1

movwf	display_1

movlw	0x98

movwf	freq_2

movwf	display_2

movlw	0x00

movwf	freq_3

movwf	display_3

;

; read reference oscillator value from EEPROM

;

banksel	Bank2		; Select Bank 2

clrf	EEADR			; Reset EEPROM read address to zero

call	read_EEPROM		; get Osc 0th byte

movwf	osc_0			; save it

call	read_EEPROM		; get Osc 1st byte

movwf	osc_1			; save it

call	read_EEPROM		; get Osc 2nd byte

movwf	osc_2			; save it

call	read_EEPROM		; get Osc 3rd byte

movwf	osc_3			; save it

;

; set up LCD display to show 10,000.00 MHz on first line

;

call	 clr_LCD

call	 bin2BCD

call	 show_freq

movlw	 0xC7			; display "CAL" on second line

movwf	 LCD_char

call	 cmnd2LCD

movlw	 'C'			; send a C

movwf	 LCD_char

call	 data2LCD

movlw	 'A'			; send an A

movwf	 LCD_char

call	 data2LCD

movlw	 'L'			; send an L

movwf	 LCD_char

call	 data2LCD

;

movf   PORTA,w          ; Read port

movwf  ren_read     	; Save it in ren_read

movlw  0x03         	; Get encoder mask

andwf  ren_read,w   	; Get encoder bits

movwf  ren_old          ; Save in ren_old

call	 reset_DDS		; this is the power-on sequence

;

;   calibration loop

;

cal_loop

call	 calc_dds_word	; using current osc value

call	 send_dds_seq	; update AD9834

clrf   last_dir     	; Set the knob direction indicator to Down

clrf	 fstep_3		; set step size to 10 Hz

clrf	 fstep_2

clrf	 fstep_1

movlw	 0x10

movwf	 fstep_0		;

call	 poll_encoder	; wait until encoder has moved

;

update_osc

btfsc	 last_dir,1		; are we slowing down the osc?

goto	 faster		; no, increase osc value.

;

;	slow down master oscillator (slower)

;

comf	 fstep_0,f

comf	 fstep_1,f

comf	 fstep_2,f

incfsz fstep_0,f

goto	 faster

incfsz fstep_1,f

goto	 faster

incfsz fstep_2,f

;

;	speed up master oscillator (faster)

;

faster

movf	 fstep_0,w

addwf	 osc_0,f

btfss	 STATUS,B_C

goto	 add4

incfsz osc_1,f

goto	 add4

incfsz osc_2,f

add4

movf	 fstep_1,w

addwf	 osc_1,f

btfss	 STATUS,B_C

goto	 add5

incfsz osc_2,f

add5

movf	 fstep_2,w

addwf	 osc_2,f

btfss	 STATUS,B_C

add6

btfss	 PORTA,pb_2

goto	 cal_loop

;

;	the calibrate button has been released or cleared

;     so now write the new oscillator values to EEPROM

;     Note: we are only going to store updated values for

;	osc_0, osc_1, and osc_2.  Osc_3 (00) remains unchanged

;     by calibrate routine.

;

banksel Bank2		; go to bank 2

clrf	  EEADR

banksel Bank0

movf	  osc_0,w

banksel Bank2

movwf	  EEDATA

call	  write_EEPROM

movf	  osc_1,w

banksel Bank2

movwf	  EEDATA

call	  write_EEPROM

movf	  osc_2,w

banksel Bank2

movwf	  EEDATA

call	  write_EEPROM

banksel Bank2

clrf	  EEADR

banksel Bank0

call	  clr_LCD

call	  init_LCD

call	  display_version

return

;*****************************************************************************

;   Subtract routine, added 4/29/04 to provide IF frequency offset adjustment

;	to frequency display.

;

;*****************************************************************************

subtract

clrf	display_0		; clear out old values before each loop through

clrf	display_1		; this routine

clrf	display_2

clrf	display_3

movf	freq_0,w		; move low byte subtrahend to w

sublw	minuend_0		; subtract w from low byte minuend literal

movwf	display_0		; save result in display_0

movlw	minuend_1		; move next byte minuend literal to w

btfss	STATUS,B_C		; If carry bit is set

addlw	0xFF			; add a -1

movwf	display_1		; temp save carry-corrected m_1 in display_1

movf	freq_1,w		; move next byte subtrahend to w

subwf	display_1,w		; subtract from adjusted minuend m_1

movwf	display_1		; save final result in display_l

movlw	minuend_2		; move next minuend literal to w

btfss	STATUS,B_C		; if carry bit is set

addlw	0xFF			; add a -1

movwf	display_2		; temp save carry-corrected m_2 in display_2

movf	freq_2,w		; move next subtrahend to w

subwf	display_2,w		; subtract from corrected m_2, save in w

movwf	display_2		; save final result in display_2

movlw	minuend_3		; move hi byte minuend literal to w

btfss	STATUS,B_C		; if carry bit is set

addlw	0xFF			; add a -1

movwf	display_3		; temp save carry_correcte m_3 in display_3

movf	freq_3,w		; move hi byte subtrahend to w

subwf	display_3,w		; subtract from corrected m_3

movwf	display_3		; save result in display_3

return

;

;

; *****************************************************************************

; * init_LCD routine                                                          *

; * Purpose:  Power on initialization of Liquid Crystal Display.  The LCD     *

; *           controller chip must be equivalent to an Hitachi 44780.  The    *

; *           LCD is assumed to be a 16 X 2 display. Also includes a clear_LCD*

; *           subroutine to clear out the LCD after init, used in calibrate.  *

; *   Input:  None                                                            *

; *                                                                           *

; *  Output:  None                                                            *

; *                                                                           *

; *****************************************************************************

;

init_LCD

call    wait_64ms         ; Wait for LCD to power up

movlw   0x30              ; LCD init instruction (First)

;		Put 4-bit command in RB7...RB4

;		PIC RB7...RB4 ports connect to LCD DB7...DB4 (pins 14-11)

movwf   PORTB             ; Send to LCD via RB7..RB4

bsf     PORTB,LCD_e       ; Set the LCD E line high,

call    wait_128ms        ;   wait a "long" time,

bcf     PORTB,LCD_e       ;   and then Clear E

movlw   0x30              ; LCD init instruction (Second)

movwf   PORTB             ; Send to LCD via RB7..RB4

bsf     PORTB,LCD_e       ; Set E high,

call    wait_128ms        ;   wait a long while,

bcf     PORTB,LCD_e       ;   and then Clear E

movlw   0x30              ; LCD init instruction (Third)

movwf   PORTB             ; Send to LCD via RB7..RB4

bsf     PORTB,LCD_e       ; Set E high,

call    wait_128ms        ;   wait a long while,

bcf     PORTB,LCD_e       ;   and then Clear E

movlw   0x20              ; 4-bit mode instruction

movwf   PORTB             ; Send to LCD via RB7..RB4

bsf     PORTB,LCD_e       ; Set E high,

call    wait_32ms         ;   wait a while,

bcf     PORTB,LCD_e       ;   and then Clear E

movlw   0x28              ; 1/16 duty cycle, 5x8 matrix

call    cmnd2LCD          ; Send command in w to LCD

movlw   0x08              ; Display off, cursor and blink off

call    cmnd2LCD          ; Send command to LCD

clr_LCD

movlw   0x01              ; Clear and reset cursor

call    cmnd2LCD          ; Send command in w to LCD

movlw   0x06              ; Set cursor to move right, no shift

call    cmnd2LCD          ; Send command in w to LCD

movlw   0x0C              ; Display on, cursor and blink off

call    cmnd2LCD          ; Send command in w to LCD

return                    ;

;

; ****************************************************************************P

; * display_version routine                                                   P

; * Purpose:  Display version and other info on LCD for 2 seconds             P

; *           upon power-up                                                   P

; *                                                                           P

; *   Input:  MCODE_REV_0 through MCODE_REV_3 set up                          P

; *                                                                           P

; *  Output:  LCD displays debug info                                         P

; *                                                                           P

; ****************************************************************************P

;

display_version

movlw   0x80              ; Point LCD at char 0

call    cmnd2LCD          ;

movlw   'W'               ; Char 0

call    data2LCD          ;

movlw   '6'               ; Char 1

call    data2LCD          ;

movlw   'I'               ; Char 2

call    data2LCD          ;

movlw   'B'			; Char 3

call	data2LCD		;

movlw   'C'               ; Char 4

call    data2LCD          ;

movlw   ' '               ; Char 5

call    data2LCD          ;

movlw   'L'               ; Char 6

call	data2LCD 	      ;

movlw	'P'			; Char 7

call    data2LCD          ;

movlw	'D'			; Char 8

call	data2LCD		;

movlw	'D'			; Char 9

call	data2LCD		;

movlw	'S'			; Char 10

call	data2LCD		;

movlw	' '			; Char 11

call	data2LCD		;

movlw	'V'			; Char 12

call	data2LCD		;

movlw	'F'			; Char 13

call	data2LCD		;

movlw	'O'			; Char 14

call 	data2LCD		;

call    wait_64ms         ; Wait one second

movlw   b'11000000'       ; Point LCD at 1st char, second line

movwf   LCD_char          ;

call    cmnd2LCD          ;

movlw   'V'               ; Clear char 0

call    data2LCD          ;

movlw   'e'               ; Char 1

call    data2LCD          ;

movlw   'r'               ; Char 2

call    data2LCD          ;

movlw   ' '               ; Char 3 (space)

call    data2LCD          ;

movlw   MCODE_REV_0	      ;Get mcode rev byte

call    data2LCD          ;   and display it

movlw   MCODE_REV_1       ; Get mcode rev byte

call    data2LCD          ;   and display it

movlw   MCODE_REV_2       ; Get mcode rev byte

call    data2LCD          ;   and display it

movlw   MCODE_REV_3       ; Get mcode rev byte

call    data2LCD          ;   and display it

call    wait_a_sec        ; Wait one second

call	clr_LCD			  ; clear off the LCD screen

return

;

; *****************************************************************************

; *  bin2BCD routine                                                          *

; * Purpose:  This subroutine converts a 32 bit binary number to a 10 digit   *

; *           BCD number.  The input value taken from freq(0 to 3) is         *

; *           preserved.  The output is in BCD(0 to 4), each byte holds =>    *

; *           (hi_digit,lo_digit), most significant digits are in BCD_4.      *

; *           This routine is a modified version of one described in          *

; *           MicroChip application note AN526.                               *

; *                                                                           *

; *   Input:  The value in freq_0 ... freq_3                                  *

; *                                                                           *

; *  Output:  The BCD number in BCD_0 ... BCD_4                               *

; *                                                                           *

; *****************************************************************************

;

bin2BCD

movlw   0x20              ; Set loop counter

movwf   BCD_count         ;   to 32

clrf    BCD_0             ; Clear output

clrf    BCD_1             ;   "     "

clrf    BCD_2             ;   "     "

clrf    BCD_3             ;   "     "

clrf    BCD_4             ;   "     "

bin_loop

bcf     STATUS,B_C        ; Clear carry bit in STATUS

;

; Rotate bits in freq bytes.  Move from LS byte (freq_0) to next byte (freq_1).

; Likewise, move from freq_1 to freq_2 and from freq_2 to freq_3.

;

;	These are the frequency bytes sent to DDS, not adjusted for IF offset

;		MUST BE DISABLED AND COMMENTED OUT WHEN IF-OFFSET MODE IS IMPLEMENTED

;    rlf     freq_0,f          ; Rotate left, 0 -> LS bit, MS bit -> Carry

;    rlf     freq_1,f          ; Rotate left, Carry->LS bit, MS bit->Carry

;    rlf     freq_2,f          ; Rotate left, Carry->LS bit, MS bit->Carry

;    rlf     freq_3,f          ; Rotate left, Carry->LS bit, MS bit->Carry

rlf	 display_0		 ; these are the IF offset adjusted bytes

rlf	 display_1		 ; that work with Subtract subroutine

rlf	 display_2		 ; now enabled

rlf	 display_3

btfsc   STATUS,B_C        ; Is Carry clear? If so, skip next instruction

;    bsf     freq_0,0          ; Carry set so wrap and set bit 0 in freq_0

;		MUST BE COMMENTED OUT WHEN IF OFFSET MODE IS IMPLEMENTED

bsf	 display_0,0

;

; Build BCD bytes. Move into LS bit of BCD bytes (LS of BCD_0) from MS bit of

; freq_3 via the Carry bit.

;

rlf     BCD_0,f           ; Rotate left, Carry->LS bit, MS bit->Carry

rlf     BCD_1,f           ; Rotate left, Carry->LS bit, MS bit->Carry

rlf     BCD_2,f           ; Rotate left, Carry->LS bit, MS bit->Carry

rlf     BCD_3,f           ; Rotate left, Carry->LS bit, MS bit->Carry

rlf     BCD_4,f           ; Rotate left, Carry->LS bit, MS bit->Carry

decf    BCD_count,f       ; Decrement loop count

btfss   STATUS,B_Z        ; Is loop count now zero?

goto    adjust            ; No, go to adjust

return                    ; Yes, EXIT

; ============================================================================

adjust  ; Internal subroutine, called by bin2BCD main loop only

;

; As BCD bytes are being built, make sure the nibbles do not grow larger than 9.

; If a nibble gets larger than 9, increment to next higher nibble.

; (If the LS nibble of a byte overflows, increment the MS nibble of that byte.)

; (If the MS nibble of a byte overflows, increment the LS nibble of next byte.)

;

movlw   BCD_0             ; Get pointer to BCD_0

movwf   FSR               ; Put pointer in FSR for indirect addressing

call    adj_BCD           ;

incf    FSR,f             ; Move indirect addressing pointer to BCD_1

call    adj_BCD           ;

incf    FSR,f             ; Move indirect addressing pointer to BCD_2

call    adj_BCD           ;

incf    FSR,f             ; Move indirect addressing pointer to BCD_3

call    adj_BCD           ;

incf    FSR,f             ; Move indirect addressing pointer to BCD_4

call    adj_BCD           ;

goto    bin_loop          ; Back to main loop of bin2BCD

; ============================================================================

adj_BCD  ; Internal subroutine, called by adjust only

movlw   3                 ; Add 3

addwf   INDF,w            ;   to LS digit

movwf   BCD_temp          ; Save in temp

btfsc   BCD_temp,3        ; Is LS digit + 3 > 7  (Bit 3 set)

movwf   INDF              ; Yes, save incremented value as LS digit

movlw   0x30              ; Add 3

addwf   INDF,w            ;   to MS digit

movwf   BCD_temp          ; Save as temp

btfsc   BCD_temp,7        ; Is MS digit + 3 > 7  (Bit 7 set)

movwf   INDF              ; Yes, save incremented value as MS digit

return                    ; Return to adjust subroutine

;

; *****************************************************************************

; * show_freq routine                                                         *

; * Purpose:  Display the frequency setting on the LCD.                       *

; *                                                                           *

; *   Input:  The values in BCD_4 ... BCD_0                                   *

; *                                                                           *

; *  Output:  The number displayed on the LCD                                 *

; *                                                                           *

; *****************************************************************************

;

show_freq

movlw   0x80              ; Point the LCD to first LCD digit location

call    cmnd2LCD          ; Send starting digit location to LCD

;

; Running 4-bit mode, so need to send Most Significant Nibble first.

;

; Extract and send "XXXX" from byte containing "XXXXYYYY"

;  - Swap halves to get YYYYXXXX

;  - Mask with 0x0F to get 0000XXXX

;  - Add ASCII bias (0030XXXX)

;

swapf   BCD_3,w           ; Swap 10MHz BCD digit into lower nibble of W

andlw   0x0F              ; Mask for lower nibble only       (0000XXXX)

addlw   0x30              ; Add offset for ASCII char set    (0030XXXX)

call    data2LCD          ; Send byte in W to LCD

;

; Extract and send "YYYY" from byte containing "XXXXYYYY"

;   - Mask with 0x0F to get 0000YYYY

;   - Add offset for ASCII character set in LCD  (0030YYYY)

;

movf    BCD_3,w           ; Put 1MHz BCD digit into lower nibble of W

andlw   0x0F              ; Mask for lower nibble only       (0000YYYY)

addlw   0x30              ; Add offset for ASCII char set    (0030YYYY)

call    data2LCD          ; Send byte in W to LCD

;

movlw   ','               ; Get a comma

call    data2LCD          ; Send byte in W to LCD

;

swapf   BCD_2,w           ; Swap 100KHz BCD digit into lower nibble of W

andlw   0x0F              ; Mask for lower nibble only       (0000XXXX)

addlw   0x30              ; Add offset for ASCII char set    (0030XXXX)

call    data2LCD          ; Send byte in W to LCD

;

movf    BCD_2,w           ; Put 10KHz BCD digit into lower nibble of W

andlw   0x0F              ; Mask for lower nibble only       (0000YYYY)

addlw   0x30              ; Add offset for ASCII char set    (0030YYYY)

call    data2LCD          ; Send byte in W to LCD

;

swapf   BCD_1,w           ; Swap 1KHz BCD digit into lower nibble of W

andlw   0x0F              ; Mask for lower nibble only       (0000XXXX)

addlw   0x30              ; Add offset for ASCII char set    (0030XXXX)

call    data2LCD          ; Send byte in W to LCD

;

movlw   '.'               ; Set up W with ASCII Period

call    data2LCD          ; Send data byte in W to LCD

;

;   movlw   0xC0              ; Point to LCD digit number nine

;   call    cmnd2LCD          ; Send command byte in W to LCD

;

movf    BCD_1,w           ; Put 100 Hz BCD digit into lower nibble of W

andlw   0x0F              ; Mask for lower nibble only       (0000YYYY)

addlw   0x30              ; Add offset for ASCII char set    (0030YYYY)

call    data2LCD          ; Send data byte in W to LCD

;

swapf   BCD_0,w           ; Swap 10 Hz BCD digit into lower nibble of W

andlw   0x0F              ; Mask for lower nibble only       (0000XXXX)

addlw   0x30              ; Add offset for ASCII char set    (0030XXXX)

call    data2LCD          ; Send data byte in W to LCD

;

movf    BCD_0,w           ; Put 1 Hz BCD digit into lower nibble of W

andlw   0x0F              ; Mask for lower nibble only       (0000YYYY)

addlw   0x30              ; Add offset for ASCII char set    (0030YYYY)

call    data2LCD          ; Send byte in W to LCD

;

movlw   ' '               ; Send a space

call    data2LCD          ;   to LCD

;

movlw   'k'               ; Send a 'k'

call    data2LCD          ;   to LCD

;

movlw   'H'               ; Send an "H"

call    data2LCD          ;   to LCD

;

movlw   'z'               ; Send a 'z'

call    data2LCD          ;   to LCD

;

return                    ;

;

; *****************************************************************************

; *  busy_check routine                                                       *

; * Purpose:  Check if LCD is done with the last operation.                   *

; *           This subroutine polls the LCD busy flag to determine if         *

; *           previous operations are completed.                              *

; *                                                                           *

; *   Input:  None                                                            *

; *                                                                           *

; *  Output:  PORTB set as:  RB7..RB4 inputs                                  *

; *                          RB3..RB0 outputs                                 *

; *****************************************************************************

;

busy_check

movlw	b'00000001'

movwf	PORTB			  ; clear Port B except for bit 0 which must

; stay high for DDS control

bsf     STATUS,B_RP0      ; Switch to bank 1 for Tristate operation

movlw   b'11110000'       ; Set RB7..RB4 as inputs, RB3..RB0 outputs

movwf   TRISB             ;   via Tristate

bcf     STATUS,B_RP0      ; Switch back to bank 0

bcf     PORTB,LCD_rs      ; Set up LCD for Read Busy Flag (RS = 0)

bsf     PORTB,LCD_rw      ; Set up LCD for Read (RW = 1)

movlw   0xFF              ; Set up constant 255

movwf   timer1            ;   for timer loop counter

LCD_is_busy

bsf     PORTB,LCD_e       ; Set E high

movf    PORTB,w           ; Read PORTB into W

movwf   LCD_read          ; Save W for later testing

bcf     PORTB,LCD_e       ; Drop E again

nop                       ; Wait a

nop                       ;   while

bsf     PORTB,LCD_e       ; Pulse E high (dummy read of lower nibble),

nop                       ;   wait,

bcf     PORTB,LCD_e       ;   and drop E again

decf    timer1,f          ; Decrement loop counter

btfsc   STATUS,B_Z        ; Is loop counter down to zero?

goto    not_busy          ; If yes, return regardless

btfsc   LCD_read,7        ; Is Busy Flag (RB7) in save byte clear?

goto    LCD_is_busy       ; If not, it is busy so jump back

not_busy

return                    ;

;

; *****************************************************************************

; * cmnd2LCD routine										*

; * Purpose:  Send Command or Data byte to the LCD                            *

; *           Entry point cmnd2LCD:  Send a Command to the LCD                *

; *           Entry Point data2LCD:  Send a Data byte to the LCD              *

; *                                                                           *

; *   Input:  W has the command or data byte to be sent to the LCD.           *

; *                                                                           *

; *  Output:  None                                                            *

; *****************************************************************************

;

cmnd2LCD   ; ****** Entry point ******

movwf   LCD_char          ; Save byte to write to LCD

clrf    rs_value          ; Remember to clear RS  (clear rs_value)

bcf     PORTB,LCD_rs      ; Set RS for Command to LCD

goto    write2LCD         ; Go to common code

data2LCD   ; ****** Entry point ********

movwf   LCD_char          ; Save byte to write to LCD

bsf     rs_value,0        ; Remember to set RS (set bit 0 of rs_value)

bsf     PORTB,LCD_rs      ; Set RS for Data to LCD

write2LCD

call    busy_check        ; Check to see if LCD is ready for new data

movlw	b'00000001'

movwf	PORTB			; clear Port B except for bit 0 (DDS sync)

bsf     STATUS,B_RP0      ; Switch to bank 1 for Tristate operation

movlw   0x00              ; Set up to enable PORTB data pins

movwf   TRISB             ; All pins (RB7..RB0) are back to outputs

bcf     STATUS,B_RP0      ; Switch to bank 0

bcf     PORTB,LCD_rw      ; Set LCD back to Write mode  (RW = 0)

bcf     PORTB,LCD_rs      ; Guess RS should be clear

btfsc   rs_value,0        ; Should RS be clear?  (is bit 0 == 0?)

bsf     PORTB,LCD_rs      ; No, set RS

;

; Transfer Most Significant nibble  (XXXX portion of XXXXYYYY)

;

movlw   0x0F              ; Set up mask

andwf   PORTB,f           ; Clear old RB7..RB4

movf    LCD_char,w        ; Put byte of data into W

andlw   0xF0              ; Mask to give XXXX0000 in W

iorwf   PORTB,f           ; Send to RB7..RB4 without changing RB3..RB0

bsf     PORTB,LCD_e       ; Pulse the E line high,

nop                       ;   wait,

bcf     PORTB,LCD_e       ;   and drop it again

;

; Transfer Least Significant nibble  (YYYY portion of XXXXYYYY)

;

movlw   0x0F              ; Set up mask

andwf   PORTB,f           ; Clear old RB7..RB4

swapf   LCD_char,w        ; Move LS nibble of data to MS position in W

andlw   0xF0              ; Mask to give YYYY0000 in W

iorwf   PORTB,1             ; Send to RB7..RB4 without changing RB3..RB0

bsf     PORTB,LCD_e       ; Pulse the E line high,

nop                       ;   wait,

bcf     PORTB,LCD_e       ;   and drop it again

return

;

; *********************************************************************

;  write_EEPROM routine (Adapted for PIC16F819)

;  Purpose:  Write the byte of data at EEdata to the EEPROM at address

;            EEADR.

;

;    Input:  The values already written to EEdata and EEADR.

;

;   Output:  The EEPROM value is updated and EEADR is incremented

;			by one.

;

; **********************************************************************

;

write_EEPROM

write_EEPROM

banksel	Bank3		    ; Switch to bank 3

bcf	EECON1, EEPGD   ; point to EEPROM data memory

bsf     EECON1,WREN     ; Set the EEPROM write enable bit

bcf	INTCON,GIE	    ; disable Interrupts

movlw   0x55            ; Write 0x55 and 0xAA to EEPROM

movwf   EECON2          ;   control register, as required

movlw   0xAA            ;   for the write

movwf   EECON2          ;

bsf     EECON1,WR       ; Set WR to initiate write

bsf     INTCON,GIE	    ; reenable interrupts

bcf	EECON1, WREN    ; disable EEPROM writes

btfsc	EECON1, WR	    ; wait for write to complete

goto	$-1

banksel	Bank2

incf    EEADR,f	    ; increment EE write address

banksel	Bank0		    ; go back to bank 0

return

;

; ***************************************************************************

;   read_EEPROM routine 	(Adapted for PIC16F8819)

;  Purpose:  Read a byte of EEPROM data at address EEADR into EEdata.

;

;    Input:  The address has been previously moved from W into EEADR

;

;   Output:  The value in EEDATA is placed into W register and

; 			 the address is incremented by one

;

; *****************************************************************************

;

read_EEPROM

banksel Bank3		; switch to bank 3

bcf	 EECON1,EEPGD	; Point to data memory

bsf     EECON1,RD  	; Request the read

banksel Bank2		; switch to bank 2

movf	 EEDATA,w 	      ; move data from EEDATA to W reg

incf    EEADR,f          ; Increment the read address

banksel Bank0		; switch to Bank 0

return                	; Return to the caller

;

;

; *****************************************************************************

; * Delay loops                                                               *

; * Purpose:  Wait for a specified number of milliseconds.                    *

; *										    								  *

; *           Entry point for wait_a_second                                   *

; *           Entry point for wait_256 msec					 			      *

; *           Entry point wait_128ms:  Wait for 128 msec                      *

; *           Entry point wait_64ms :  Wait for 64 msec                       *

; *           Entry point wait_32ms :  Wait for 32 msec                       *

; *           Entry point wait_16ms :  Wait for 16 msec                       *

; *           Entry point wait_8ms  :  Wait for 8 msec                        *

; *                                                                           *

; *   Input:  None                                                            *

; *                                                                           *

; *  Output:  None                                                            *

; *                                                                           *

; *****************************************************************************

;

wait_a_sec  ; ****** Entry point *****

call wait_256ms

call wait_256ms

call wait_256ms

call wait_256ms

return

wait_256ms  ; ****** Entry point ******

call wait_128ms

call wait_128ms

return

wait_128ms  ; ****** Entry point ******

movlw   0xFF              ; Set up outer loop

movwf   timer1            ;   counter to 255

goto    outer_loop        ; Go to wait loops

wait_64ms  ; ****** Entry point ******

movlw   0x80              ; Set up outer loop

movwf   timer1            ;   counter to 128

goto    outer_loop        ; Go to wait loops

wait_32ms   ; ****** Entry point ******

movlw   0x40              ; Set up outer loop

movwf   timer1            ;   counter to 64

goto    outer_loop        ; Go to wait loops

wait_16ms   ; ****** Entry point ******

movlw   0x20              ; Set up outer loop

movwf   timer1            ;   counter to 32

goto    outer_loop        ; Go to wait loops

wait_8ms   ; ****** Entry point ******

movlw   0x10              ; Set up outer loop

movwf   timer1            ;   counter to 16

; Fall through into wait loops

;

; Wait loops used by other wait routines

;  - 1 microsecond per instruction (with a 4 MHz microprocessor crystal)

;  - 510 instructions per inner loop

;  - (Timer1 * 514) instructions (.514 msec) per outer loop

;  - Round off to .5 ms per outer loop

;

outer_loop

movlw   0xFF              ; Set up inner loop counter

movwf   timer2            ;   to 255

inner_loop

decfsz  timer2,f          ; Decrement inner loop counter

goto    inner_loop        ; If inner loop counter not down to zero,

;   then go back to inner loop again

decfsz  timer1,f          ; Yes, Decrement outer loop counter

goto    outer_loop        ; If outer loop counter not down to zero,

;   then go back to outer loop again

return                    ; Yes, return to caller

;

;

END